| Source | Sector | Region | Nodes | Intertemporal | GHG reduction (relative to 1990 level) | Cost Deviation |
| --- | --- | --- | --- | --- | --- | --- |
| Price et al. | Coupled | Global | 16 | Yes | 50 % | < 10 % |
| DeCarolis et. al. | Electricity | US | 1 | Yes | 80 % | < 15 % |
| Li et. al. | Electricity | UK | 1 | Yes | 80 % | < 15 % |
| Sasse et al. | Electricity | CH | 2,258 | No | None | < 20 % |
| Trutnevyte et al. | Electricity | UK | 1 | No | None | < 23 % |
| Nacken et al. | Coupled | DE | 1 | No | None | < 10 % |
| Neumann et al. | Electricity | Europe | 100 | No | 100 % | < 10 % |
| My study | Electricity | DE | 1 | Yes | 95% | User Defined |

- First I want to start with motivation of my study

- Why it worths studying this topic, what is the point

- Classical cost optimization of an energy system returnas a single cost optimal solution.

- However long term investment decisions of an energy systems can not be made purely cost driven

- There are many other criterias that remain uncaptured by the model

- For example, social acceptance, scale of infrastructure projects, complexity and scale of the system itself,

- Those criterias can be counted as structural uncertainties of the model

- In addition to that, input parameters to the model are highly uncertain by nature

- Long term prediction of Cost of technologies, fuel prices, load profile

- As an end effect ,Real world transitions are usually more costly than modelled cost optimal solutions

- For this reasons it is important to analyse near cost optimal alternatives of an energy system.

- But what exactly near cost optimal alternatives

Near Cost Optimal alternatives are similar costly but sub cost optimal alternative solutions to cost optimal solution.

- In my study I utilize Modelling to Generate Alternatives  methodology to study near optimal space of German power system.

- In this methodology  cost optimal solution is used as an anchor point to explore the surrounding decision space

- In this graph this point is PV capacity given by the cost optimal solution as we diverge from cost optimum we see investment flexibility increases. 2% cost increase gives almost 50 GW flexibility in PV capacity investment

- So why modeling tio generate alternatives

- MGA helps to

- Observe technologically diverse but similar costly solutions

- Offers alternative transition pathways to meet unmodelled criteria like social exceptance for example

- Evaluate the stability of cost optimal solution			, here we seeoffered PV capacity by  cost optimum solution is not stable at all

- Show persisterent long term investment decisions		,again here we can say that even with 15% cost increase it is not possible to retrackt from PV technology completely

- Now as we discussed about the methodology applied lets talk about the novel point. W

- What is the novel point of this study?

- Here you see a list of studies that apply Studies that apply modeling to generate alternatives (MAG) to energy system models.

- To my  knowledge,  my study is the first to utilize MGA for an intertemporal model of German power system.

- I am going to analyze the period from 2020 to 2050 with total of 4 different modelled years.

- The model will also consider progressive GHG mitigation goal defined by the German authorities.

- To  put it in one sentence

- Goal of my thesis is to explore near-optimal, intertemporal investment alternatives to German power system model while sticking to long term emmission goals of Energiewende.

- Hmm! now probably you are asking yourselves what is the deal with intertemporal?

- Well, Intertemporal models analyse the development of the power system through a modeling horizon.

- They give the solution as snapshots of the transition across multiple years

- And plus they consider the investment dependencies between modelled years

- On the left side you see a single year solution, which gives cost optimum for 2034

- On the right side you see the same solution with 4 different moddelled years from 2019 to 2034.

- In the latter it is possible to observe how the transition will look like.

- As we are finished with terminology lets take a look at the application procedure

- I applied MAG on „URBS“ , which is a linear energy system modelling framework

- URBS already has working intertemporal modules, however it is originally capable of doing only classical optimization.

- In order to apply MAG with urbs I created an independent module that analyses near cost optimal alternatives

- Which first runs the model with a classical cost optimization.

- Minimum cost found is applied to the model as a new constraint with a small percentage increase

- A set of generation capacities are choosen to be objected to optimization   , here only PV capacity is choosen but program is designed to accept also more than one

- A new objective function is formulated to first maximize then minimize objected generation capacities

- As outcome a range of capacity is obtained for the same cost

- Some other capabilites of the new feature:

- It takes a list of slack values and applies them in succession automatically

- For multi node models it is possible to choose optimization regions independently for each optimized generation technology.

- Region selection  made available also for the original objectives

- Now it is time to see the first results

- I have the input file for 2050 German power system from a former student‘s master Thesis

- I analyzed this input file my new module

- In the first graph PV capacity is subjected to optimization

- and in the second graph onshore wind capacity is subjected

- As we see ,

- for current ghg mitigation goals it is not possible to imagine a power system for germay either without onshore wind or PV technologies

- However as we diverge from the minimum cost, investment flexibility of technologies improves drastically.

- PV capacity has a greater range of flexibility as also pointed out by other similar studies

- Here is an intertemporal solution.

- Example input files from urbs git hub repository is used for this analysis

- On left side only solution of the last year is shown and

- on the right side you see development of the solution through years

- These graphs are only for pv capacities, which is the objected technology

- Here is another view of the same solution this time only for 10 percent cost increse

- On the left side you see how system will evolve when pv capacity is subjected to minimization

- On the right side PV capacity is subjected to maximization

- Here we can interpret that wind energy and PV are substitutes of each other

- Minimization of one will cause increase of the other due to ambitious GHG reduction goals.

- Lastly I want to talk about the next steps I plan to achieve during course of my thesis